| Solar Condition | Standard | Power Density (Wm-2) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Total | 250 - 2500 nm | 250 - 1100 nm |
|  | WMO Spectrum | 1367 |  |  |
| AM 0 | ASTM E 490 | 1353 | 1302.6 | 1006.9 |
| AM 1 | CIE Publication 85, Table 2 |  | 969.7 | 779.4 |
| AM 1.5 D | ASTM E 891 | 768.3 | 756.5 | 584.7 |
| AM 1.5 G | ASTM E 892 | 963.8 | 951.5 | 768.6 |
| AM 1.5 G | CEI/IEC* 904-3 | 1000 | 987.2 | 797.5 |

The most widely used standard spectra are those published by The Committee Internationale d'Eclaraige (CIE), the world authority on radiometric and photometric nomenclature and standards. The American Society for Testing and Materials (ASTM) publish three spectra - the AM 0, AM 1.5 Direct and AM 1.5 Global for a 37° tilted surface. The conditions for the AM 1.5 spectra were chosen by ASTM "because they are representative of average conditions in the 48 contiguous states of the United States". Fig. 5 shows typical differences in standard direct and global spectra. These curves are from the data in ASTM Standards, E 891 and E 892 for AM 1.5, a turbidity of 0.27 and a tilt of 37° facing the sun and a ground albedo of 0.2.