ECE 2212

PROBLEM SET 11

Due: FRIDAY, 21 April 2017

S. G. Burns

NOTE 1:	I will not collect Problem Set for grading, however you are well-prepared for Quiz 11, Friday 21 April, if you bring your completed Problem Set 11 with you!

NOTE 2:  Quiz 11 is scheduled for Friday, 21 April, not a Wednesday, 19 April.

- Text 15.112.  Also sketch and label the back-to-back load lines.

- From an old quiz.

Assume a Class B  audio amplifier operating between 48 volts  and driving a 4  speaker.  Neglect the effect of crossover distortion.

- Provide required maximum ratings as indicated for the transistors using a well-labeled sketch of the back-to-back load lines to illustrate your answers.

IC(max)  = ____________		VCE(max) = ___________		PC(max) = __________

- Design a protection circuit to limit the transistor to the IC(max)  you computed in part (a)  should the amplifier output be short-circuited.    Incorporate this protection circuit in your basic Class B circuit diagram.

3.	Amplifier Analysis

A partial circuit diagram for a Fujitsu MB 47358 operational amplifier is given below.  Assume V+ = 15 volts and V- = -15 volts.  For the npn transistors, assume = 150, VBE(on) = 0.7 Volts, and VAN = 100 Volts.  For the pnp transistors, assume = 100, VBE(on) = 0.55 Volts, and VAP = 50 Volts.

(a)	Briefly describe the circuit  function/operation  of :

Q10 and Q11

Q13 and Q14 (Diode connected transistors)

Q7 Circuit topology

Q5 and Q6

(b)	Assume a sinusoidal input voltage that results in an output voltage vo(t) = 12 sin (t) volts.  RL = 1 k.

Compute values for :

- Peak and average power to the load resistor.

- Resultant collector efficiency.

- Power required from the power supplies for the output stage.

- A design value for RSC to limit the short circuited load current to 30 mA .

(c )	Estimate the following :

- Collector currents in    Q5 and Q6

- Collector currents in  Q1 and Q2

- Voltage at the base of Q5 and Q6

- Voltage at the emitter of Q7